SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL2- COMPARATIVE ANALYSIS |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Bandari Lakshitha Sankar Reddy(192571085) | Reg.No.: | 192571085 |
| Department: | CSE -BIO SCI | Date: | 27.08.26 |

ANNEXURE A

COMPARATIVE ANALYSIS

1.Compare L1, L2, and L3 cache levels in terms of latency, bandwidth, and throughput, and analyze their impact on processor performance.

2.Differentiate SRAM and DRAM by evaluating their latency, bandwidth, and throughput characteristics in cache and main memory design.

3.Compare virtual memory with paging and cache memory mechanisms in terms of access latency and system throughput.

4.Analyze the differences between NAND Flash and DRAM with respect to latency, bandwidth, and suitability for hierarchical memory systems.

5.Compare MRAM and RRAM technologies by examining their performance in terms of access time, throughput, and energy efficiency.

Code of Conduct:

I Bandari Lakshitha Sankar Reddy(192571085) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Bandari Lakshitha sankar Reddy(192571085)

Signature of the student:

MARKS ALLOCATION

|  | Scope of Items (20%) | Comparison Depth (25%) | Use of Evidence (20%) | Critical Thinking (20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

1. Comparison of L1, L2 and L3 Cache Levels

Cache memory is a small and high-speed memory located close to the processor. It stores frequently or recently accessed instructions and data so that the CPU does not always have to access slower main memory. Modern processors generally use a hierarchical cache structure consisting of L1, L2 and L3 caches.

The major difference between these levels is the trade-off between capacity, latency, bandwidth and sharing. L1 is the smallest and fastest, while L3 is larger but slower. Intel describes L1 as the shortest-latency cache and L3/LLC as the last and longest-latency cache before main memory.

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Position | Closest to CPU core | After L1 | Before main memory |
| Size | Smallest | Medium | Largest |
| Latency | Lowest | Higher than L1 | Higher than L2 |
| Bandwidth | Very high | High | Lower than L1/L2 |
| Typical use | Immediate CPU operations | Backup for L1 | Shared backup for cores |
| Usually shared? | Generally private | Often private/shared depending on design | Commonly shared |
| Effect of miss | Goes to L2 | Goes to L3 | Goes to DRAM |

L1 Cache

L1 is the first cache checked by the processor. It is generally divided into instruction cache (L1-I) and data cache (L1-D). Because it is located very close to the execution units and is relatively small, it provides the lowest access latency.

For example, Intel documentation describes L1 as the first and shortest-latency level in the memory hierarchy.

Intel provides an illustrative memory-performance comparison in which an L1 cache has about 1 ns latency and around 1 TB/s bandwidth, although actual values vary significantly by processor generation and workload.

A high L1 hit rate is therefore extremely important. If a processor repeatedly finds required data in L1, it can continue executing with minimal waiting.

L2 Cache

L2 is larger than L1 but normally has somewhat greater latency. It acts as a second level of protection against expensive accesses to main memory.

When data is not found in L1, the processor checks L2. An L2 hit is much faster than going to DRAM, but it still causes more delay than an L1 hit. Intel notes that avoiding L1 misses that subsequently hit in L2 can improve latency and overall performance.

L3 Cache

L3 is generally the largest cache and is commonly shared among multiple processor cores. It is slower than L1 and L2 but is still much faster than accessing DRAM.

In multicore processors, L3 is especially useful because one core may benefit from data that is already available in the shared last-level cache. Intel describes L3/LLC as the last cache level before DRAM and notes that its latency can be significantly greater than L2.

Impact on processor performance

The cache hierarchy improves processor performance by reducing the average memory access time. A simplified relationship is:

Average Memory Access Time ≈ Hit Time + Miss Rate × Miss Penalty

Therefore, a high L1 hit rate can dramatically reduce the time spent waiting for memory.

For example, consider a processor executing a program that repeatedly uses the same array elements. If those elements remain in L1, the processor can access them quickly. If the data is not in L1 but is found in L2, the processor experiences a slightly greater delay. If it misses L1, L2 and L3, it may have to access DRAM, producing a much larger delay.

Thus, L1 provides minimum latency, L2 provides additional capacity without excessive latency, and L3 reduces expensive DRAM accesses in multicore systems.

Intel also provides examples where newer processor architectures increase L2 capacity to improve hit rates and reduce effective memory latency.

Conclusion: The three cache levels are not competing technologies; they work together. L1 prioritizes speed, L2 balances speed and capacity, and L3 prioritizes capacity and sharing. A well-designed hierarchy therefore increases CPU throughput and reduces processor stalls.

2. Difference Between SRAM and DRAM

SRAM (Static Random Access Memory) and DRAM (Dynamic Random Access Memory) are both volatile memory technologies, but they differ significantly in construction, speed, density and application.

SRAM commonly uses multiple transistors to store each bit, while DRAM uses a transistor-capacitor structure. Because SRAM does not require the same refresh mechanism as DRAM, it can provide very fast access.

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full form | Static RAM | Dynamic RAM |
| Typical access time | Around 1 ns | Around 10 ns |
| Refresh required | No | Yes |
| Density | Lower | Higher |
| Cost per bit | Higher | Lower |
| Power | Higher cell area/cost; no refresh | Requires refresh |
| Main application | CPU cache | Main memory |
| Volatile | Yes | Yes |
| Typical cell | Multi-transistor | 1 transistor + capacitor |

A technology comparison reports approximately 1 ns read/write time for SRAM and approximately 10 ns for DRAM, illustrating the significant latency advantage of SRAM.

SRAM in cache memory

SRAM is well suited to cache memory because the processor needs extremely fast access. The small size of SRAM cells allows cache circuits to operate at high speed.

However, SRAM requires considerably more chip area per stored bit than DRAM. This makes very large SRAM caches expensive and physically difficult to implement.

Therefore, processors use relatively small SRAM caches such as L1 and L2 instead of using SRAM for the entire main memory.

DRAM in main memory

DRAM has much higher storage density than SRAM and is less expensive per bit. This makes it appropriate for large-capacity main memory.

However, DRAM stores information using electrical charge that gradually leaks away. Consequently, the memory must periodically perform refresh operations.

The additional refresh requirement and memory-array structure contribute to greater latency compared with SRAM.

Bandwidth and throughput

Bandwidth represents how much data can be transferred per unit time, while latency represents how long it takes for an individual request to begin receiving useful data.

SRAM has an advantage in latency and is highly suitable for repeated processor accesses. DRAM, however, can provide very high aggregate bandwidth when organized using wide memory buses and multiple channels.

Therefore, the comparison cannot simply be stated as “SRAM is faster in every situation.” SRAM is primarily optimized for low access latency, while DRAM provides a much better capacity-to-cost ratio and can deliver substantial aggregate memory bandwidth.

Example

Suppose a CPU repeatedly accesses a small lookup table.

If the lookup table fits inside SRAM-based cache, the CPU can access it with very low latency.

If the same table were stored only in DRAM, the CPU would experience higher access latency. However, DRAM allows the system to store gigabytes of data at a reasonable cost.

Thus, the two technologies are complementary:

SRAM → high speed, small capacity → cache

DRAM → lower speed, large capacity → main memory

Conclusion: SRAM is preferred for cache because latency is the dominant requirement, while DRAM is preferred for main memory because capacity, density and cost are more important. This division forms an essential part of hierarchical memory design.

3. Virtual Memory with Paging vs Cache Memory

Virtual memory and cache memory both improve system operation, but they solve different problems.

Cache memory attempts to reduce the latency between the CPU and frequently accessed data.

Virtual memory allows programs to use an address space larger than the immediately available physical RAM and provides memory-management and isolation mechanisms.

IBM explains that virtual memory allows an operating system to operate beyond the physical memory limitations by using storage as an extension of memory.

| Feature | Cache Memory | Virtual Memory with Paging |
| --- | --- | --- |
| Main purpose | Reduce memory access latency | Extend/manage memory address space |
| Managed mainly by | Hardware | OS + MMU |
| Unit | Cache line | Page |
| Fast storage | SRAM cache | DRAM/RAM |
| Slow backing storage | Lower cache levels/DRAM | SSD/HDD |
| Typical hit | Very fast | Page already in RAM |
| Miss/fault | Cache miss | Page fault |
| Performance impact | Usually small penalty | Page fault can be very large |
| Main objective | CPU performance | Memory capacity and protection |

Cache mechanism

When the CPU requests data, the cache is checked first. If the requested information is present, a cache hit occurs.

If it is absent, a cache miss occurs and the system searches a lower level of the memory hierarchy.

The purpose is to exploit locality of reference:

- Temporal locality: recently used data is likely to be used again.

- Spatial locality: nearby data is likely to be accessed soon.

For example, when a program processes an array sequentially, nearby elements can be loaded into a cache line and reused efficiently.

Virtual memory and paging

Virtual memory divides the virtual address space into fixed-size pages. Physical memory is divided into corresponding frames.

The operating system maintains page mappings so that a virtual address can be translated to a physical location.

If a required page is already in RAM, access can proceed normally after address translation.

If the required page is not in physical memory, a page fault occurs. The operating system must obtain the page from secondary storage and load it into RAM.

IBM's documentation describes paging space as storage used for information that is part of virtual memory but is not currently resident in RAM.

Latency comparison

Cache memory has extremely low latency compared with virtual-memory page faults.

A cache hit can be serviced within a few processor cycles depending on the cache level and processor architecture.

A page fault is much more expensive because it can require interaction with storage. Even modern SSDs are substantially slower than DRAM.

Consequently:

Cache hit → very low latency

Cache miss → higher latency

Page hit in RAM → relatively low latency

Page fault → very high latency

Effect on throughput

Cache memory usually increases CPU throughput by preventing processor stalls.

Virtual memory can increase overall system flexibility and allow more applications to run simultaneously. However, excessive page faults can reduce throughput significantly.

A severe condition called thrashing can occur when the system spends a large amount of time moving pages between RAM and storage instead of executing useful instructions. IBM specifically notes that frequent page faults can contribute to thrashing.

Critical analysis

Cache and virtual memory should not be considered substitutes.

For example, a computer may have:

CPU → L1 → L2 → L3 → DRAM → SSD

Cache reduces access time within the upper part of the hierarchy, whereas virtual memory manages the relationship between virtual addresses, physical memory and backing storage.

Therefore, cache primarily improves speed, while virtual memory primarily provides capacity, protection and address-space flexibility.

Conclusion: Cache memory is designed for high-speed processor access, whereas virtual memory with paging is designed to efficiently manage memory beyond physical RAM. Cache misses normally cause relatively small delays, while page faults can produce extremely large delays and significantly reduce system throughput.

4. NAND Flash vs DRAM

NAND Flash and DRAM are both important memory technologies, but they are designed for very different purposes.

DRAM is volatile and optimized for fast working memory, while NAND Flash is non-volatile and optimized for persistent, high-density storage.

| Feature | DRAM | NAND Flash |
| --- | --- | --- |
| Volatility | Volatile | Non-volatile |
| Typical read time | ~10 ns | Much higher; technology-dependent |
| Write time | ~10 ns | Much higher, often μs–ms |
| Refresh | Required | Not required |
| Density | High | Very high |
| Endurance | Very high | Limited program/erase cycles |
| Main use | Main memory | SSDs, memory cards, storage |
| CPU working memory | Excellent | Unsuitable as direct replacement for DRAM |
| Data retention | Requires power | Retains data without power |

A review comparing memory technologies reports approximately 10 ns read/write times for DRAM, while NAND Flash is typically orders of magnitude slower for write operations.

DRAM

DRAM is used as the primary working memory of computers.

When a program is running, its active instructions and data are normally stored in DRAM because it provides much lower latency than persistent storage.

However, DRAM loses its contents when power is removed.

NAND Flash

NAND Flash is a non-volatile technology. It retains information even when the system is powered off.

It is commonly used in:

- SSDs

- USB drives

- Memory cards

- Smartphones

- Embedded storage

NAND Flash provides excellent storage density. A technology survey reports NAND Flash read times around the microsecond range in some technology comparisons, while DRAM operates around the nanosecond range.

The large difference between nanoseconds and microseconds means that NAND Flash is not normally suitable as a direct replacement for DRAM in the processor’s main-memory role.

Latency and bandwidth

DRAM has substantially lower access latency. NAND Flash has higher latency because data is accessed through flash-memory structures and controllers.

However, NAND-based SSDs can achieve high sequential throughput by using multiple flash channels and parallelism.

This distinction is important:

DRAM → low latency + high working-memory bandwidth

NAND Flash → high density + persistence + good storage throughput

Therefore, a storage device can have high sequential read/write throughput while still having much greater access latency than DRAM.

Suitability for hierarchical memory systems

A typical hierarchy can be represented as:

CPU Registers → L1 → L2 → L3 → DRAM → SSD/NAND Flash

DRAM is positioned close to the processor because the CPU needs fast access to active programs and data.

NAND Flash is placed lower in the hierarchy because it provides persistent storage rather than ultra-low-latency working memory.

Example

When a computer starts an application, the program may initially be stored on an SSD using NAND Flash.

When the application is launched, the operating system loads required code and data into DRAM.

The CPU then uses its cache hierarchy to keep frequently accessed portions close to the processor.

This demonstrates how NAND Flash and DRAM cooperate rather than directly compete.

Conclusion: DRAM is suitable for main memory because of its low latency and high bandwidth, whereas NAND Flash is ideal for persistent, high-density storage. NAND Flash provides persistence and capacity, while DRAM provides the speed required for active computation.

5. Comparison of MRAM and RRAM

MRAM (Magnetoresistive Random Access Memory) and RRAM/ReRAM (Resistive Random Access Memory) are emerging non-volatile memory technologies intended to overcome some limitations of conventional memory systems.

Both retain data without continuous power and have potential applications in embedded systems, edge computing and artificial intelligence.

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full form | Magnetoresistive RAM | Resistive RAM |
| Data storage mechanism | Magnetic state | Resistance state |
| Volatile? | No | No |
| Read speed | Very fast | Very fast |
| Write speed | Fast | Potentially very fast |
| Endurance | Extremely high | High but device-dependent |
| Energy | Low standby power | Potentially very low write energy |
| Density | Good | Very high potential |
| Major advantage | Speed + endurance | Density + low energy + multilevel potential |
| Major challenge | Write energy/current and density | Variability and endurance |
| Applications | Embedded memory, cache research | AI accelerators, embedded memory, CIM |

A recent review of STT-MRAM describes it as a non-volatile technology combining speed, endurance and density, with applications including embedded memory and potential last-level-cache use.

MRAM

MRAM stores information using magnetic states rather than electrical charge.

One major advantage is that it is non-volatile. Data can remain stored even when power is removed.

MRAM also provides excellent endurance. One review reports MRAM read times around tens of nanoseconds in one comparison, while other demonstrated technologies can achieve much faster values depending on the MRAM implementation.

A major strength of MRAM is its extremely high endurance. A technology comparison reports endurance values of around 1015cycles for STT-MRAM in representative technology data.

This makes MRAM attractive for applications where memory is frequently written.

RRAM

RRAM stores information by changing the resistance of a material.

A low-resistance state can represent one binary value, while a high-resistance state can represent another. Some RRAM devices can support multiple resistance states, making them attractive for high-density and analog/in-memory computing applications.

A review reports RRAM read and write times below 10 ns in representative technologies.

RRAM can also have very small cell sizes and potentially high density. It is therefore attractive for future memory systems and compute-in-memory (CIM) architectures.

Recent research highlights RRAM-based computing-in-memory as a possible way to reduce the data-movement bottleneck in AI workloads and improve energy efficiency.

Access time

Both technologies can provide fast access.

Representative technology comparisons show RRAM with read times around 1 ns in some device demonstrations, while MRAM is often in the few-to-tens of nanoseconds range depending on the specific implementation.

However, these values should not be interpreted as universal specifications because MRAM and RRAM include many different device architectures.

Throughput

Throughput depends not only on the intrinsic memory-cell switching speed but also on:

- Memory-array architecture

- Number of banks

- Data width

- Parallelism

- Controller design

- Read/write ratio

- Interconnect bandwidth

RRAM is especially interesting for high-throughput AI applications because many cells can potentially perform parallel operations.

MRAM, on the other hand, is attractive for conventional memory applications where high endurance and fast access are important.

Energy efficiency

Both technologies can reduce energy consumption compared with some conventional approaches, particularly because they are non-volatile and can retain data without continuous refresh.

RRAM is especially promising for low-energy AI and in-memory computing. Research surveys report very low write-energy values for RRAM devices in certain implementations.

MRAM also has excellent standby characteristics because it does not require refresh to retain information. However, the energy required for writing can be an important design consideration, particularly for some STT-MRAM implementations.

Critical comparison

The choice between MRAM and RRAM depends on the application.

If endurance and reliable repeated writing are the highest priorities, MRAM is highly attractive.

If high density, multilevel storage and in-memory computing are the priorities, RRAM can be more attractive.

Recent research describes MRAM development toward embedded non-volatile memory and potentially last-level cache, while RRAM research is strongly connected with high-density integration and AI computing-in-memory.

Conclusion: MRAM provides an excellent combination of non-volatility, speed and endurance, whereas RRAM offers strong potential in density, low-energy operation and parallel/in-memory computing. Neither technology completely replaces the other; instead, each is suitable for different levels and workloads within future hierarchical memory systems.