SIMATS ENGINEERING

Department of Computer Science & Engineering

# ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL1- INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | Bandari Lakshitha Sankar Reddy(192571085) | Reg.No.: | 192571085 |
| Department: | CSE -BIO SCI | Date: | 31.08.26 |

ANNEXURE A

# INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization

  - mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

- Cloud Server Memory Bottleneck

  - cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

- Autonomous Vehicle Data Processing

An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

- AI Inference Edge Device

An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

- High-Performance Gaming Console

  - gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

Code of Conduct:

I  Bandari Lakshitha Sankar Reddy (192571085) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Bandari Lakshitha Sankar Reddy (192571085)

Signature of the student:

# MARKS ALLOCATION

|  | Understanding of Industry Problem (20%) | Application of Engineering Concepts (25%) | Solution Design (25%) | Use of Tools (15%) | Analysis (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Industry Problem | 20% | Comprehensiv e understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding ; some requirements unclear | Poor understanding ; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementatio n | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |

Q1. Smartphone Performance Optimization

Industry Problem

A smartphone manufacturer observes lag while switching between multiple applications. When users move from one application to another, the processor must access instructions and data from different memory locations. If frequently used data is not available in cache, the processor has to access DRAM, which has considerably higher latency than cache memory.

The main objective is therefore to reduce memory access latency, improve data availability and increase processor throughput while maintaining reasonable power consumption.

Application of Engineering Concepts

Modern smartphone processors use a hierarchical memory system consisting of:

CPU Core → L1 Cache → L2 Cache → L3/Last-Level Cache → DRAM → Storage

- L1 cache: Smallest and fastest cache. It stores frequently accessed instructions and data.

- L2 cache: Larger than L1 and acts as the next level when an L1 miss occurs.

- L3 cache: Larger shared cache that can serve multiple processor cores.

- DRAM: Much larger but slower working memory used for running applications.

When switching applications, the processor may need to retrieve data that is not currently present in the higher-level caches. Frequent cache misses increase memory stalls and reduce responsiveness.

Proposed Hierarchical Memory Redesign

The smartphone can use the following optimized structure:

Application Processor
↓
L1 Instruction/Data Cache – very low latency
↓
L2 Cache – larger private/per-core cache
↓
Large Shared L3 Cache – improved inter-core data sharing
↓
High-bandwidth LPDDR DRAM
↓
UFS/NAND Flash Storage

The L1 cache should prioritize extremely low latency. L2 should provide a balance between capacity and speed, while a sufficiently large shared L3 cache can reduce expensive DRAM accesses.

The DRAM should use a modern high-bandwidth mobile memory technology such as LPDDR5/LPDDR5X, depending on the product generation and power constraints.

Use of Tools

Performance can be evaluated using:

- Android Studio Profiler

- CPU performance counters

- Cache-miss monitoring

- Memory bandwidth benchmarks

- Application switching benchmarks

Metrics such as L1/L2/L3 cache miss rate, DRAM bandwidth, memory latency and CPU utilization can be measured before and after the redesign.

Analysis

Suppose the original smartphone has a high L3 miss rate. Increasing the effective last-level cache capacity can allow more application data to remain close to the CPU.

For example:

| Parameter | Existing System | Proposed System |
| --- | --- | --- |
| L1 cache | Small | Optimized |
| L2 cache | Moderate | Increased |
| L3 cache | Limited | Larger shared cache |
| DRAM | Standard LPDDR | High-bandwidth LPDDR |
| Cache misses | High | Reduced |
| App switching | Laggy | Faster |
| CPU stalls | High | Lower |

The redesign does not simply mean “make every cache larger.” Larger caches consume more chip area and can themselves increase access time and power. Therefore, the best solution is to balance cache capacity with latency and power consumption.

Final Solution

The recommended solution is a multi-level cache hierarchy with optimized L1/L2 caches, a larger shared L3 cache and high-bandwidth LPDDR DRAM.

This reduces the number of DRAM accesses, decreases memory stalls and improves application switching responsiveness.

Expected result: Lower effective memory latency, higher processor throughput and smoother multitasking.

Q2. Cloud Server Memory Bottleneck

Industry Problem

A cloud service provider experiences high latency during database queries. Database applications frequently access indexes, records, transaction data and metadata. If the required information is not available in CPU caches or DRAM, additional memory or storage accesses can significantly increase query response time.

The problem must therefore be analyzed at two levels:

- Cache hierarchy

- Virtual memory and paging

Application of Engineering Concepts

A cloud server typically follows:

CPU → L1 → L2 → L3 → DRAM → SSD/NAND Storage

The CPU caches reduce the latency of frequently accessed instructions and database metadata.

Virtual memory provides each process with a virtual address space. The operating system maps virtual pages to physical memory frames.

If a required page is not present in physical memory, a page fault occurs. The operating system must retrieve it from storage, creating a large delay compared with a cache hit or DRAM access.

Cache vs Virtual Memory

| Aspect | Cache Hierarchy | Virtual Memory/Paging |
| --- | --- | --- |
| Main purpose | Reduce CPU-memory latency | Manage memory/address space |
| Fastest storage | SRAM cache | DRAM |
| Slow backing level | DRAM | SSD/HDD |
| Managed by | Hardware | OS + MMU |
| Miss condition | Cache miss | Page fault |
| Performance penalty | Relatively small | Potentially very high |
| Main benefit | CPU performance | Memory management |

Proposed Solution

The cloud provider should first determine whether the problem is caused by:

- High CPU cache misses

- Insufficient DRAM

- Excessive page faults

- Database working set larger than physical RAM

- Poor database locality

- Storage bottlenecks

A practical redesigned hierarchy is:

CPU
↓
Large/high-performance L1/L2 caches
↓
Large shared L3 cache
↓
High-capacity DDR5 DRAM
↓
NVMe SSD

The server should have sufficient DRAM to keep the active database working set in physical memory. This reduces paging activity.

Database software should also use efficient caching for frequently accessed records and indexes.

Use of Tools

Useful tools include:

- Linux perf

- vmstat

- free

- Database performance monitoring tools

- CPU hardware performance counters

- Memory bandwidth benchmarks

Important measurements include:

Cache miss rate + DRAM utilization + page faults + disk I/O + query response time

Analysis

Consider two database requests.

Case 1 – Cache hit

CPU requests data → L1/L2/L3 contains data → data is returned quickly.

Case 2 – Page fault

CPU requests data → cache miss → DRAM does not contain required page → page fault → OS retrieves page from SSD → page loaded into DRAM → execution continues.

The second process is much slower.

Therefore, simply increasing CPU cache cannot solve every database latency problem. If the database working set is larger than available DRAM, the system may still experience paging.

Recommended Improvements

- Increase DRAM capacity.

- Use high-bandwidth DDR5 memory.

- Increase effective last-level cache where appropriate.

- Reduce unnecessary paging.

- Optimize database indexes and queries.

- Use fast NVMe SSD for unavoidable storage operations.

- Monitor cache misses and page faults continuously.

Final Solution

The most important improvement is to keep the active database working set in DRAM and minimize page faults. Cache optimization should complement this strategy.

The recommended hierarchy is:

CPU → L1/L2 → L3 → Large DDR5 DRAM → NVMe SSD

This combination reduces CPU memory latency, minimizes paging and improves database query throughput.

Q3. Autonomous Vehicle Data Processing

Industry Problem

An autonomous vehicle continuously receives data from multiple sensors such as:

- Cameras

- LiDAR

- Radar

- Ultrasonic sensors

- GPS

- Inertial sensors

This data must be processed in real time. A delay in processing sensor information can affect object detection, path planning and vehicle control.

Therefore, the memory system must provide very low latency, high bandwidth and predictable performance.

Comparison of SRAM, DRAM and MRAM

| Feature | SRAM | DRAM | MRAM |
| --- | --- | --- | --- |
| Volatile | Yes | Yes | No |
| Speed | Very high | High | High |
| Capacity | Low | High | Medium/high potential |
| Refresh | No | Yes | No |
| Main strength | Very low latency | Large capacity | Non-volatility + endurance |
| Suitable use | Processor cache | Main memory | Fast persistent/embedded memory |

Proposed Memory Hierarchy

A suitable autonomous vehicle architecture is:

AI/CPU/GPU/NPU
↓
SRAM L1/L2 Cache
↓
Large SRAM/Last-Level Cache
↓
High-bandwidth DRAM
↓
MRAM for selected persistent/embedded workloads
↓
Non-volatile storage

Role of SRAM

SRAM should store frequently accessed information such as:

- Active sensor-processing instructions

- Frequently used neural-network data

- Real-time control variables

- Frequently accessed lookup tables

Its low latency makes it appropriate for time-critical processing.

Role of DRAM

Large sensor datasets cannot all fit into SRAM.

Therefore, high-capacity DRAM should store:

- Camera frames

- LiDAR point clouds

- Radar data

- Neural-network intermediate data

- Mapping information

High-bandwidth memory access is particularly important because multiple sensors may generate data simultaneously.

Role of MRAM

MRAM is non-volatile and has high endurance. It can be used for selected embedded applications where retaining information without power is beneficial.

For example, configuration data, frequently updated parameters or safety-related information could potentially use MRAM.

Proposed Data Flow

Sensors → DRAM buffer → SRAM cache → AI accelerator → Decision → Vehicle control

Sensor data is initially buffered in DRAM. Frequently processed portions are moved closer to the processor into SRAM.

Use of Tools

The design can be evaluated using:

- MATLAB/Simulink

- NVIDIA Nsight or equivalent accelerator profiling tools

- Linux perf

- Memory bandwidth benchmarks

- Sensor-data replay systems

Measurements should include:

- Sensor-to-decision latency

- Memory bandwidth

- Cache hit rate

- DRAM utilization

- CPU/GPU utilization

- Energy consumption

Analysis

Suppose four cameras simultaneously generate high-resolution frames. Storing all active data in SRAM would be extremely expensive because SRAM has low density.

Using only DRAM, however, could increase latency for repeatedly accessed information.

Therefore, the best architecture combines both:

SRAM → speed

DRAM → capacity and bandwidth

MRAM → non-volatility and endurance

Final Solution

The recommended architecture uses SRAM for time-critical cache data, high-bandwidth DRAM for large sensor datasets and MRAM for selected non-volatile embedded workloads.

This balances latency, capacity, bandwidth and energy consumption.

The result is a memory hierarchy capable of supporting real-time sensor fusion and AI inference while minimizing processing delays.

Q4. AI Inference Edge Device

Industry Problem

An edge AI device must load and execute machine-learning models quickly while consuming very little power.

Examples include:

- Smart cameras

- Industrial monitoring devices

- Drones

- Smart security systems

- Wearable devices

- IoT gateways

The system needs both persistent storage for the AI model and fast working memory for inference.

Comparison of NAND Flash, DRAM and RRAM

| Feature | NAND Flash | DRAM | RRAM |
| --- | --- | --- | --- |
| Volatile | No | Yes | No |
| Latency | High | Low | Very low potential |
| Capacity | Very high | High | High potential |
| Main purpose | Persistent storage | Working memory | Emerging high-speed non-volatile memory |
| Power when retaining data | Very low | Requires power/refresh | Very low |
| AI suitability | Model storage | Model execution | Potential AI acceleration |

Proposed Hierarchical Structure

The optimized structure is:

AI Accelerator/NPU
↓
On-chip SRAM/cache
↓
RRAM-based accelerator memory / high-speed memory
↓
DRAM
↓
NAND Flash

Role of NAND Flash

NAND Flash should store the complete AI model permanently.

For example, a device may store:

- Neural-network weights

- Firmware

- Configuration files

- Previous model versions

Its non-volatility means the model remains available even after the device is powered off.

However, NAND Flash has substantially higher latency than DRAM, so it should not be used as the primary working memory during inference.

Role of DRAM

DRAM stores active model data during inference.

For example:

NAND → DRAM → SRAM/NPU

The model is loaded from NAND into DRAM before execution.

DRAM provides much faster access than NAND and has sufficient capacity for large models.

Role of RRAM

RRAM is particularly interesting for AI because its resistance-based storage can support high-density non-volatile memory and, in suitable architectures, compute-in-memory.

Instead of continuously transferring weights between memory and processor, some operations can potentially be performed closer to where the weights are stored.

This can reduce the data-movement bottleneck.

Use of Tools

The design can be tested using:

- AI benchmarking frameworks

- TensorFlow Lite

- ONNX Runtime

- Hardware performance counters

- Power measurement tools

- Memory bandwidth tests

Measurements include:

- Model loading time

- Inference latency

- Frames per second

- DRAM bandwidth

- Energy per inference

- Power consumption

Analysis

Consider a smart camera running an object-detection model.

If the model is repeatedly loaded from NAND during inference, latency and power consumption increase.

Instead:

NAND stores model permanently → DRAM loads model → RRAM/SRAM keeps frequently used weights → NPU performs inference

This reduces unnecessary data movement.

Final Solution

The recommended hierarchy is:

NAND Flash → DRAM → RRAM/SRAM → AI Accelerator

NAND provides persistence, DRAM provides high-capacity working memory, and RRAM can potentially provide low-power, high-density memory close to the AI computation.

This design improves model loading, inference speed and energy efficiency.

Q5. High-Performance Gaming Console

Industry Problem

A gaming console experiences frame drops while loading large game scenes.

Modern games continuously load:

- Textures

- 3D models

- Animation data

- Audio

- Physics information

- World/level data

If these assets cannot be supplied to the CPU/GPU quickly enough, the graphics processor may remain idle or wait for memory transfers, causing frame-time spikes and frame drops.

L3 Cache vs DRAM

| Feature | L3 Cache | DRAM |
| --- | --- | --- |
| Location | Close to CPU | Main memory |
| Capacity | Smaller | Much larger |
| Latency | Lower | Higher |
| Bandwidth | Very high for cache access | Very high aggregate bandwidth |
| Main purpose | Frequently accessed data | Large working datasets |
| Best use | Hot/repeated data | Large game assets |

Role of L3 Cache

L3 cache is useful for frequently accessed CPU-side data.

Examples include:

- Game logic

- Physics calculations

- Object metadata

- Frequently accessed instructions

- Small data structures

If the required information is found in L3, the CPU avoids a DRAM access.

However, L3 cache is not large enough to contain entire high-resolution game scenes.

Role of DRAM

DRAM provides the large capacity required for active game data.

For example, a large game environment may contain gigabytes of textures and assets. These cannot realistically all be stored in CPU cache.

Therefore, DRAM provides the capacity and bandwidth needed by the CPU/GPU subsystem.

Proposed Memory Hierarchy

CPU/GPU
↓
L1 Cache
↓
L2 Cache
↓
Large Shared L3 Cache
↓
High-bandwidth GDDR/Unified DRAM
↓
NVMe SSD/NAND Storage

The exact memory technology depends on the console architecture.

Scene Loading Process

A typical scene-loading process is:

SSD → DRAM → GPU/CPU caches → Rendering engine

The SSD provides persistent game storage.

Large scene assets are loaded into DRAM.

Frequently accessed CPU data can then be cached in L1/L2/L3.

The GPU receives required resources through its high-bandwidth memory path.

Use of Tools

The manufacturer can use:

- GPU profiling tools

- CPU performance counters

- Frame-time analysis

- Memory bandwidth monitors

- Game-engine profiling tools

Important measurements include:

Average FPS + 1% low FPS + frame time + cache miss rate + DRAM bandwidth + asset-loading time

Analysis

Suppose a game normally renders at 60 FPS.

The target frame time is approximately:

Frame time = 1/60 ≈ 16.67 ms

If a large scene-loading operation causes the CPU/GPU to wait for memory, the frame time might temporarily rise above this value.

For example, if frame time increases to 30 ms, the effective instantaneous frame rate is approximately:

FPS = 1000 / 30 ≈ 33.3 FPS

This produces visible stuttering.

Increasing L3 cache can reduce CPU-side memory misses, but it cannot solve every scene-loading problem because large textures and other assets still require substantial DRAM bandwidth and storage throughput.

Therefore, both cache and DRAM must be optimized.

Proposed Enhancements

- Increase effective L3 cache capacity where beneficial.

- Optimize L2/L3 cache utilization.

- Use high-bandwidth DRAM.

- Improve memory-channel utilization.

- Use fast NVMe storage.

- Stream assets asynchronously.

- Compress and decompress game assets efficiently.

- Prefetch assets before they are required.

- Reduce unnecessary memory transfers between CPU and GPU.

Final Solution

The most effective solution is not simply increasing L3 cache. Instead, the console should use:

Optimized L1/L2/L3 cache + high-bandwidth DRAM + fast NVMe storage + intelligent asset prefetching

L3 reduces latency for frequently accessed CPU data, while DRAM supplies the large volume of data required for rendering.

This combination reduces frame-time spikes and improves gaming smoothness.